DE MARTIN & GASPARINI - Quality Assurance System

Inspection and Test Checklist

| BUILDER : LendLease |  |  |  |  | PROJECT NO : J0805 |  |  | DOC NO :QA Rev 2 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PROJECT: One Circular Quay |  |  |  |  |  |  |  | DATE: 8.01.24 |  |  |  |  |  |  |  |
| DESCRIPTION : Concrete Supply and Place |  |  |  |  |  |  |  | AUSTRALIAN STANDARDS: AS3600 / AS1379 / AS1012 |  |  |  |  |  |  |  |
| SITE STRUC. SPECS: 18295-RBG-ZZ-XX-SP-ST-00001[01] |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| LOCATION : As per attached markup drawings |  |  |  |  |  |  |  | DATE CONCRETED: 8.01.24 |  |  |  |  |  |  |  |
| SPECIFIED MIX (MPa): |  |  | SLAB: WALLS: COLUMNS: |  |  |  |  |  |  |  |  |  |  |  |  |
| PLACEMENT METHOD : PUMP/TRUCK/KIBBLE/OTHER |  |  |  |  |  |  |  |  |  |  | DMG SUPERVISOR: Daniel Rossetto |  |  |  |  |
| CONCRETE POURED (Element / Volume / MPa): |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (TA) 80MPa - 3m3- Level 8 Pour 2 L7-L8 Columns |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (TA) 65MPa - 7m3- Level 8 Pour 2 L7-L8 Wall & slab puddling |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (TA) 50MPa - 15m3- Level 8 Pour 2 L7-L8 Wall |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| (TA) 40MPa - 57m3- Level 8 Pour 2 slab |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| WEATHER : |  | Fine |  |  |  | TOTAL m3 : |  |  | 82m3 |  |  |  |  |  |  |
| TIME START: |  | 07:30 |  |  |  | TIME FINISH : |  |  | 5:30 |  |  |  |  |  |  |
| ITP | Activity |  |  |  |  | Acceptance Criteria |  |  |  |  |  | INITIALS | DATE |  | REVIEW |
|  | PRE-POUR ITP INSPECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Determine equipment requirements: - Pumps & Delivery lines - Placing tools - Finishing Machines |  |  |  |  | Visual |  |  |  |  |  | S.V. | 8.01.24 |  | W= Witness |
| 2 | Main Slab RL |  |  |  |  | Drawing No. |  |  |  |  |  | S.V. | 8.01.24 |  | W |
| 3 | Ensure Pre-pour inspection complete and signed |  |  |  |  | Yes / No |  |  |  |  |  | S.V. | 8.01.24 |  | W |
| 4 | Check cleanliness of slab /formed walls & columns - clear of debris |  |  |  |  | Visual |  |  |  |  |  | S.V. | 8.01.24 |  | W |
| 5 | Received formwork Certificate |  |  |  |  | Yes / No |  |  |  |  |  | S.V. | 8.01.24 |  | H = Hold |
| 6 | Specific slab finish requirement steel trowel |  |  |  |  | Drawing No. |  |  |  |  |  | S.V. | 8.01.24 |  | W |
| 7 | Check concrete mix Required? Y/N |  |  |  |  | Use approved mix design. |  |  |  |  |  | S.V. | 8.01.24 |  | W |
| 8 | Check concrete mix design and strengths |  |  |  |  | Mix designs to meet the requirements of AS 1379 as specified in the approved Boral Mix Design Submission. In accordance with RBG concrete supply specification. |  |  |  |  |  | S.V. | 8.01.24 |  | W |
| 9 | Concrete adequately vibrated |  |  |  |  | Visual |  |  |  |  |  | S.V. | 8.01.24 |  | W |
| 10 | Concrete tester in place (Testing as per Engineer Specification) |  |  |  |  | Compressive strength testing as per AS1012. In accordance with RBG concrete testing specification. |  |  |  |  |  | S.V. | 8.01.24 |  | W |
|  | POST-POUR ITP INSPECTION |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 1 | Application of curing agents |  |  |  |  | Masterkure250 – conforms to AS 3799 & in accordance with RBG Concrete Place & Finishing Specification |  |  |  |  |  | S.V. | 8.01.24 |  | W |
| 2 | Post pour inspection – check slab finish/quality – clean up |  |  |  |  | Visual |  |  |  |  |  | S.V. | 8.01.24 |  | W |
| 3 | Pre-Stress As Built Survey – complete and submit to Lendlease at earliest opportunity |  |  |  |  | Visual |  |  |  |  |  | S.V. | 8.01.24 |  | W |
| Final Acceptance |  |  |  | Organisation |  |  | Name |  |  | Signature |  |  |  | Date |  |
|  |  |  |  | DMG |  |  | Sal Volpato |  |  |  |  |  |  | 8.01.24 |  |
|  |  |  |  | LendLease |  |  |  |  |  |  |  |  |  |  |  |

## Revision 2, July 2023	Prepared & approved by Sal Volpato

| 0-i;ft a |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | t1,.1J1inln,lh)ql,11g. |  |  |  |  |
| .,,.,. ui,e1. |  |  |  | - ,nu | •1D11t:W•••• |  |  |
| Qn«1r•• "'914 |  |  |  |  |  |  |  |
| Q4 1 |  |  | ,Q,-i1419 lilil,l,,rjl•■ ,.., OI\LI _.Hil1 |  |  |  |  |
| \Mµ'IN,,1 '1 ' 1 |  |  |  |  |  |  |  |
| Rov |  | f 0.do "'-" |  | D.l!JiUllpllllfl! |  | CIPF "'- | I RO!JNO |

GENEIIAl ARRANGEMENT LEGEND:

Q	Drnom1u ITUCOIIC ETEElEMENTOVER

.	CJ

EI]

-·

-	c:c:,

DEtlOTESLOAD R111C[LfMWTUllO£ A/10 IU!JIU COIKRETE ElEMEUTOVfR

li. lh'.ft.o'\lDIW00it41"

'"'(_RobertBirdGroup

Mem rollntSurtM1n•Jura111Group

®	-I­	'

IIB?  OEN OTES !t_<XKl•IO K AIIO TYi'!; r::z:::z::.u, PEfERTOORAWIUCS OSHA·N 1001-01

RBl  TO BSTTAN IOOIOHOR:.CHtDULE

=.,"AIIOTYPICALOETAILS

EZl

IZI

[2]

OWOTES>Or11mWETAAEA SETDOWII

Sl tl'UIW.EIIHEII

':':•R:!o n'!U.t.n1G0q1111111"r'1'r'u1t 1o1Gs,.,,,_mill,l..l,i,,l.l..

hd,,..., .a.-.i,_,. l'h•	,,..rt_,.,.

.,11,1_ :., :·

©---I

L-=-.:_  -- J,:. iti_

[-	.	- l I

-

- -·

-	-	-l

Ilic.

'inai' M• l{lIJI .	'_

I	I	-

I

-	I

I	...-..!.!"-\")11<1	I

I

THIC'IMwt

Drnomsu.amP

e

8

SIC

•1'1-.«ill(.-JJilitW UIIJIHLHIKlEIWIIU:11611

.11Wmu:1nowut11

>E,"1Hll\UU◄llfCl5

JllLaU\151/U;"I

-

Dtl/OTTSVBT1'1Crn!SS

-

: :• '!;; 51fHT.

U

"'r"Df:"Wn".1\'fI.C.,i,'! ''"'

---1 -

OCNOTES COLUMN IIUMBER AND92E

'wl

Of!lOTESWALLNUMf:f.R& OIRECTIOIIELEVATIVIIISVlf'IIED

CRONE

li.;; L!•:•;1,0

lfllJIUI.Mll',DIIIEClfll)l',Ell9

I

110'

DENOTES400W X •)OL 500

cP(tfPl"1'-'=PS

I

,, ,,

I

I	ii : .

a

,I

I

1ZB

Q

REBA.TE

DENOTISPl:UETRATIONSMIO CAST IN SERVICf'ifRU ZONE

(1WOTESSTAWMASSCON•:RETE

><E,oo

'A"N"D"

.. 1J'l..'1\Sl"l

lflllll'liDll'ROUllCl(ltlN'R9

.u. ,...........,.,...I.UI.D...- IE

-li,ft•Niltlll'."1.-.Pm

==

OENOTE$10Cl#Xli'WLX95D

SOfflTRHl.t..TI

t•EIWHSCASTINUflltl• E'l'E RHER

ASSOC!AlES

-

..,.in,lf'

fl<lfP1 1{,Gfl'V'

-

·­

I

®

:	i	1

---rc;;►i

1 M	I	I

@)	j

' ly&H.,.11	,1o.'o;oo

Jfl)...	8

7T ""	r	yr-

□ II

I

-Towzt...A

TOR8STTAN100061fOROETAltS

D£NVHS 2'J[1W l'10L X 5t.JO fACAOEMLLLIL.Jl,hil-\TE

DWOTESfACAOEEME£0MEIH/CA5TIN

(iStant c

Ll10nP•ClflC>t<>1..,,1 rr.•RTHS1Drtc, fNiWl

la'IH!IIICN.£LEC1111'.111.EII.Dlll.ll

□

ST>.NTEC

L•. L " XJI PA r1c,_,

I

Irr

I

©-	I

### LL,.,,

l.,.j)

HIii	!D)...E) t_;-	•

I

jl\¥1»	I	1§10

00A500  B>OUIA

□,

L

-'$'jlfwt.?, o,;::: f,4:=-Mf"'

l,c:;V'et..-  S	po	1...

-1 1  Oy lOOMfo.

CHA!:Pl(l REfERTOC [AIIOS.:G ORAVl,IIGSfO Ci.JTAIIDDETAILS

_c±_ ornonscm1ml)(mt1Jo11n

5' c- 1% ric2HoE l ; c! :botcT ABEU(1

r,;-e;; DEN01ES6SMP PUOOLEPOOR RUERTO

l.,......d  COLUMUOETAllSs,jEEJRS.STiA)Ol(()O\I

NOH:U(EkAlCIIIHCTSDkAIVIMGSro,

,..,_...,,,

OIL!,<IAN:•S.>J;•.'{IOS

ARfllfTDR.IIJLC!HtYUSE/£lhllll,

LE\Hn,101.,Llf T.

r+:$1

11.\FTHOH•I.E!tUl:u.n

l/HlQ

'!,'; '00)

I

1 j'

Col-vtv1.N'S"	fr

·31'Vl3	F

wr,,..

Bo N(i\

ML\HPSANDrALLSINSLIII\

CONCRETE NOTES: lHEkNOTCSONCOVEkSIIUT MIIICOHC.!i"TUNGTHIITl8DAYS

....

""'""

U)O_ttoill	ft

-l)a!IIJI!""I'O,f'!'OOROO>W.t

:, :::E I L•EfSPtm>lt

@-	-+:

C-oL..,V""tJS

(,)1.1,1•<1

,._..,..

nrE WML		 ElE AllCiiS

0---:- I

- 7-vv.3

c.P	{;5(v\f

IOMPi UIIO

.............."'°"0 .. lb ... .,,	,..-.,..,-_,,u.,_-.-,.,.,._.,_

'"" 	.,... .., , .. ..".':'.-"!:.

=· -- -··_..;,-;.,--:=:= :.;3:: .

I	Ir.

[ : ,-; 1

I ll r,

\tJf,..t.l......

-t.. gLA,3

#### vs-nt cM

..:....i::- ! '.: ._!",::. ,..:--.:.:.!·'

00 NOT HALE DlAWING ,UH flGUU0DlWN\I0NS

IIITllOG[NllALNOllSUNLmNullDOlll[RI!,(

I	wf Ji -® -

ITH: <·B T TA 4•) lw.l l I

f0RLEVELl5LOCAL

THICHlllllGC•ETAIL

@-	L	Q	_  J ...,t • [	i ,  _ i _	_ ,.,,, n

I

RUERCOPEP1.J,.11sroP lll((oR(SL,1.BDITAILS

'CC,.'/\-_

nrERCOREPLAIISrOR

_L__J_	111c-,:, ES-LABD-ETA1-L

-1I.--·-· ---------+	·--

I --1 srAt:i"t·,---- w.- l!!"-;- --- .:- ..: I --=----"=	c-.

--ii.t-

" -  •- ••n •• h-•

-:::-,-_

_J	I

-- !-+!...,---	,

D,11.. ,RZ

c ""'' HA

l	i	i	i

- -	,

---

II	, ,· >-.41,.

Ilb_)J-

TOWER A LEVEL 11 GENERAL ARRANGEMENT PLAN(LEVELS 12 TO 19 SIMILAR)

,CAf.11 ti,1

e/,/2+

( -	, 	i

_....!_-1_- -:_..J)O ti) i

SC-ll.fll(XJ  -1:..1	Wll:111  'f

t

t

i

ONE CIRCULAR QUAY TOWER A

-_-,-.-,	!

LEVEL 11 GENERAL	0

ARRANGEMENT PLAN (LEVELS 12 <

TO 19 SIMILAR)

!

,IUl1A.Rat  ,	Ii!iI;

FOR CONSTRUCTION  RB-ST-TA-10-1110-01	A

# Delivery Info

Date	08/01/2024

Order	C1899750

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	8B COMPY

Truck Number	10880

# Times

Batched	08:39 AM

Left Plant	08:55 AM

Arrived on Site	09:04 AM

Started Unloading	09:31 AM

Finished Unloading	09:31 AM

Left Site	09:34 AM

Time on Site	29

Product

Type	CONCRETE CUSTOMER PRESCRIPTION ASPIRE47

Details	CONCRETE CUSTPRESC ASPIRE47 80MPA 10MM 650 SLUMP

Requested Slump	650

Mix Code	80ASPCTP45

# Quantities

This Load	3 m³

Total Progress	3 m³

Total Ordered	3 m³

# Water

Water Design	474 L

Water Batched	444 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	30 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 3 m³ |
| Product and additives 253631 | CONCRETE CUSTPRESC ASPIRE47 80MPA 10MM | 3 m³ |

## Driver Instructions

DANIEL 0423291890

# Account charged

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899759

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	9.5 KM

Truck Type	8B COMPY

Truck Number	10351

# Times

Batched	08:58 AM

Left Plant	09:07 AM

Arrived on Site	09:30 AM

Started Unloading	09:33 AM

Finished Unloading	10:01 AM

Left Site	10:04 AM

Time on Site	33

# Product

Type	CONCRETE ENVISIA HIGH SLUMP

Details	ENVISIA HIGH SLUMP 40MPA 20MM 150MM 150 SLUMP

Requested Slump	150

Mix Code	40VSTZPP15

# Quantities

This Load	6 m³

Total Progress	6 m³

Total Ordered	57 m³

# Water

Water Design	1116 L

Water Batched	1087 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	29 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees |  |  |
| 257145 | TRANSPORT CHARGE | 6 m³ |
| 231299 | WAITING TIME FEE WORKING HOURS | 10 /M |
| Product and additives |  |  |
| 256219 | ENVISIA HIGH SLUMP 40MPA 20MM 150MM | 6 m³ |

## Driver Instructions

DANIEL 0423291890

# Account charged

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899759

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	8A MAXI

Truck Number	10824

# Times

Batched	09:14 AM

Left Plant	09:25 AM

Arrived on Site	09:49 AM

Started Unloading	10:06 AM

Finished Unloading	10:17 AM

Left Site	10:20 AM

Time on Site	31

# Product

Type	CONCRETE ENVISIA HIGH SLUMP

Details	ENVISIA HIGH SLUMP 40MPA 20MM 150MM 150 SLUMP

Requested Slump	150

Mix Code	40VSTZPP15

# Quantities

This Load	5.3 m³

Total Progress	11.3 m³

Total Ordered	57 m³

# Water

Water Design	986 L

Water Batched	952 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	34 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 5.3 m³ |
| Product and additives 256219 | ENVISIA HIGH SLUMP 40MPA 20MM 150MM | 5.3 m³ |

## Driver Instructions

DANIEL 0423291890

# Account charged

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899765

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	10 WHEEL

Truck Number	10516

# Times

Batched	09:33 AM

Left Plant	09:49 AM

Arrived on Site	10:05 AM

Started Unloading	10:20 AM

Finished Unloading	10:50 AM

Left Site	10:50 AM

Time on Site	45

# Product

Type	CONCRETE HIGH STRENGTH

Details	HIGHSTRENGTH 65MPA 10MM 150 SLUMP

Requested Slump	150

Mix Code	65HS5ETPXX

# Quantities

This Load	7 m³

Total Progress	7 m³

Total Ordered	7 m³

# Water

Water Design	1358 L

Water Batched	1320 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	38 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees |  |  |
| 257145 | TRANSPORT CHARGE | 7 m³ |
| 231299 | WAITING TIME FEE WORKING HOURS | 20 /M |
| 131495 | FEE SERVICE WAITING TIME PER MINUTE | 10 /M |
| Product and additives |  |  |
| 161151 | HIGHSTRENGTH 65MPA 10MM | 7 m³ |
| Driver Instructions |  |  |
| DANIEL 0423291890 |  |  |
| Account charged |  |  |

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899759

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	10 WHEEL

Truck Number	10726

# Times

Batched	10:07 AM

Left Plant	10:19 AM

Arrived on Site	10:28 AM

Started Unloading	11:07 AM

Finished Unloading	11:07 AM

Left Site	11:07 AM

Time on Site	39

# Product

Type	CONCRETE ENVISIA HIGH SLUMP

Details	ENVISIA HIGH SLUMP 40MPA 20MM 150MM 150 SLUMP

Requested Slump	150

Mix Code	40VSTZPP15

# Quantities

This Load	7 m³

Total Progress	18.3 m³

Total Ordered	57 m³

# Water

Water Design	1302 L

Water Batched	1265 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	37 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 7 m³ |
| Product and additives 256219 | ENVISIA HIGH SLUMP 40MPA 20MM 150MM | 7 m³ |

## Driver Instructions

DANIEL 0423291890

# Account charged

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899744

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	10 WHEEL

Truck Number	10517

# Times

Batched	10:23 AM

Left Plant	10:43 AM

Arrived on Site	10:57 AM

Started Unloading	11:29 AM

Finished Unloading	11:29 AM

Left Site	11:29 AM

Time on Site	32

# Product

Type	CONCRETE ASPIRE50

Details	ASPIRE50 100MPA 10MM 650 SLUMP

Requested Slump	650

Mix Code	11ASPHIAG2

# Quantities

This Load	5 m³

Total Progress	5 m³

Total Ordered	15 m³

# Water

Water Design	770 L

Water Batched	738 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	32 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 5 m³ |
| Product and additives 245097 | ASPIRE50 100MPA 10MM | 5 m³ |
| Driver Instructions DANIEL 0423291890 |  |  |
| Account charged Account Name |  | DEMARTIN & GASPARINI-L370 |
| Customer Account Code |  | 503098 |
| Customer Name |  | DEMARTIN & GASPARINI-L370 |
| Customer Code |  | 29948 |
| Project Name |  | ONE CIRCULAR QUAY - TOWER A |
| Project Code |  | Q-00141017 |
| Purchase Order Number |  | J0805 - A |
| Abridged Conditions of Quotation and Supply - Concrete |  |  |

# Delivery Info

Date	08/01/2024

Order	C1899744

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	8A MAXI

Truck Number	10620

# Times

Batched	10:37 AM

Left Plant	10:55 AM

Arrived on Site	11:11 AM

Started Unloading	11:20 AM

Finished Unloading	11:20 AM

Left Site	11:20 AM

Time on Site	9

# Product

Type	CONCRETE ASPIRE50

Details	ASPIRE50 100MPA 10MM 650 SLUMP

Requested Slump	650

Mix Code	11ASPHIAG2

# Quantities

This Load	5 m³

Total Progress	10 m³

Total Ordered	15 m³

# Water

Water Design	770 L

Water Batched	750 L

Water Added at Plant	6 L

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	20 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 5 m³ |
| Product and additives 245097 | ASPIRE50 100MPA 10MM | 5 m³ |
| Driver Instructions DANIEL 0423291890 |  |  |
| Account charged Account Name |  | DEMARTIN & GASPARINI-L370 |
| Customer Account Code |  | 503098 |
| Customer Name |  | DEMARTIN & GASPARINI-L370 |
| Customer Code |  | 29948 |
| Project Name |  | ONE CIRCULAR QUAY - TOWER A |
| Project Code |  | Q-00141017 |
| Purchase Order Number |  | J0805 - A |
| Abridged Conditions of Quotation and Supply - Concrete |  |  |

# Delivery Info

Date	08/01/2024

Order	C1899744

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	10 WHEEL

Truck Number	10921

# Times

Batched	10:49 AM

Left Plant	10:54 AM

Arrived on Site	11:28 AM

Started Unloading	11:31 AM

Finished Unloading	12:04 PM

Left Site	12:04 PM

Time on Site	36

# Product

Type	CONCRETE ASPIRE50

Details	ASPIRE50 100MPA 10MM 650 SLUMP

Requested Slump	650

Mix Code	11ASPHIAG2

# Quantities

This Load	5 m³

Total Progress	15 m³

Total Ordered	15 m³

# Water

Water Design	770 L

Water Batched	743 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	27 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 5 m³ |
| Product and additives 245097 | ASPIRE50 100MPA 10MM | 5 m³ |
| Driver Instructions DANIEL 0423291890 |  |  |
| Account charged Account Name |  | DEMARTIN & GASPARINI-L370 |
| Customer Account Code |  | 503098 |
| Customer Name |  | DEMARTIN & GASPARINI-L370 |
| Customer Code |  | 29948 |
| Project Name |  | ONE CIRCULAR QUAY - TOWER A |
| Project Code |  | Q-00141017 |
| Purchase Order Number |  | J0805 - A |
| Abridged Conditions of Quotation and Supply - Concrete |  |  |

# Delivery Info

Date	08/01/2024

Order	C1899759

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	10 KM

Truck Type	10 WHEEL

Truck Number	10728

# Times

Batched	11:11 AM

Left Plant	11:23 AM

Arrived on Site	11:47 AM

Started Unloading	11:53 AM

Left Site	12:12 PM

Time on Site	24

# Product

Type	CONCRETE ENVISIA HIGH SLUMP

Details	ENVISIA HIGH SLUMP 40MPA 20MM 150MM 150 SLUMP

Requested Slump	150

Mix Code	40VSTZPP15

# Quantities

This Load	7 m³

Total Progress	25.3 m³

Total Ordered	57 m³

# Water

Water Design	1302 L

Water Batched	1260 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	42 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 7 m³ |
| Product and additives 256219 | ENVISIA HIGH SLUMP 40MPA 20MM 150MM | 7 m³ |

## Driver Instructions

DANIEL 0423291890

# Account charged

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899759

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	9.5 KM

Truck Type	8B COMPY

Truck Number	10008

# Times

Batched	11:35 AM

Left Plant	11:44 AM

Arrived on Site	12:02 PM

Started Unloading	12:02 PM

Finished Unloading	12:19 PM

Left Site	12:19 PM

Time on Site	17

# Product

Type	CONCRETE ENVISIA HIGH SLUMP

Details	ENVISIA HIGH SLUMP 40MPA 20MM 150MM 150 SLUMP

Requested Slump	150

Mix Code	40VSTZPP15

# Quantities

This Load	6 m³

Total Progress	31.3 m³

Total Ordered	57 m³

# Water

Water Design	1116 L

Water Batched	1078 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	38 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 6 m³ |
| Product and additives 256219 | ENVISIA HIGH SLUMP 40MPA 20MM 150MM | 6 m³ |

## Driver Instructions

DANIEL 0423291890

# Account charged

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899759

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	8B COMPY

Truck Number	10084

# Times

Batched	11:54 AM

Left Plant	12:03 PM

Arrived on Site	12:11 PM

Started Unloading	12:18 PM

Finished Unloading	12:35 PM

Left Site	12:37 PM

Time on Site	25

# Product

Type	CONCRETE ENVISIA HIGH SLUMP

Details	ENVISIA HIGH SLUMP 40MPA 20MM 150MM 150 SLUMP

Requested Slump	150

Mix Code	40VSTZPP15

# Quantities

This Load	6 m³

Total Progress	37.3 m³

Total Ordered	57 m³

# Water

Water Design	1116 L

Water Batched	1083 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	33 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 6 m³ |
| Product and additives 256219 | ENVISIA HIGH SLUMP 40MPA 20MM 150MM | 6 m³ |

## Driver Instructions

DANIEL 0423291890

# Account charged

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899759

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	10 KM

Truck Type	10 WHEEL

Truck Number	10662

# Times

Batched	12:00 PM

Left Plant	12:16 PM

Arrived on Site	12:32 PM

Started Unloading	12:36 PM

Finished Unloading	12:49 PM

Left Site	12:52 PM

Time on Site	19

# Product

Type	CONCRETE ENVISIA HIGH SLUMP

Details	ENVISIA HIGH SLUMP 40MPA 20MM 150MM 150 SLUMP

Requested Slump	150

Mix Code	40VSTZPP15

# Quantities

This Load	7 m³

Total Progress	44.3 m³

Total Ordered	57 m³

# Water

Water Design	1302 L

Water Batched	1268 L

Water Added at Plant	3 L

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	34 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 7 m³ |
| Product and additives 256219 | ENVISIA HIGH SLUMP 40MPA 20MM 150MM | 7 m³ |

## Driver Instructions

DANIEL 0423291890

# Account charged

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899759

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	8B COMPY

Truck Number	10880

# Times

Batched	12:18 PM

Left Plant	12:34 PM

Arrived on Site	12:41 PM

Started Unloading	12:48 PM

Finished Unloading	01:12 PM

Left Site	01:14 PM

Time on Site	32

# Product

Type	CONCRETE ENVISIA HIGH SLUMP

Details	ENVISIA HIGH SLUMP 40MPA 20MM 150MM 150 SLUMP

Requested Slump	150

Mix Code	40VSTZPP15

# Quantities

This Load	6 m³

Total Progress	50.3 m³

Total Ordered	57 m³

# Water

Water Design	1116 L

Water Batched	1086 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	30 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 6 m³ |
| Product and additives 256219 | ENVISIA HIGH SLUMP 40MPA 20MM 150MM | 6 m³ |

## Driver Instructions

DANIEL 0423291890

# Account charged

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899759

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	10 WHEEL

Truck Number	10921

# Times

Batched	12:38 PM

Left Plant	12:43 PM

Arrived on Site	01:04 PM

Started Unloading	01:14 PM

Finished Unloading	01:26 PM

Left Site	01:26 PM

Time on Site	22

# Product

Type	CONCRETE ENVISIA HIGH SLUMP

Details	ENVISIA HIGH SLUMP 40MPA 20MM 150MM 150 SLUMP

Requested Slump	150

Mix Code	40VSTZPP15

# Quantities

This Load	4.7 m³

Total Progress	55 m³

Total Ordered	57 m³

# Water

Water Design	874 L

Water Batched	846 L

Water Added at Plant	NOT CURRENTLY AVAILABLE

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	28 L

| Delivery Note |  |  |
| --- | --- | --- |
| Service Fees 257145 | TRANSPORT CHARGE | 4.7 m³ |
| Product and additives 256219 | ENVISIA HIGH SLUMP 40MPA 20MM 150MM | 4.7 m³ |

## Driver Instructions

DANIEL 0423291890

# Account charged

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

# Delivery Info

Date	08/01/2024

Order	C1899759

Delivery Address	1, ALFRED ST, SYDNEY

Plant	ARTARMON NSW MC

Est. Distance	7 KM

Truck Type	10 WHEEL

Truck Number	10660

# Times

Batched	01:17 PM

Left Plant	01:30 PM

Arrived on Site	01:39 PM

Started Unloading	01:44 PM

Finished Unloading	02:02 PM

# Product

Type	CONCRETE ENVISIA HIGH SLUMP

Details	ENVISIA HIGH SLUMP 40MPA 20MM 150MM 150 SLUMP

Requested Slump	150

Mix Code	40VSTZPP15

# Quantities

This Load	2 m³

Total Progress	57 m³

Total Ordered	57 m³

# Water

Water Design	372 L

Water Batched	318 L

Water Added at Plant	3 L

Water Added on Site	NOT CURRENTLY AVAILABLE

Max Water Allowable	54 L

# Delivery Note

| Service Fees |  |  |
| --- | --- | --- |
| 231407 | MINIMUM LOAD (3 M3) | 1 m³ |
| 231301 | FEE SERVICE PUMP BLOW BACK PER CU METRE | 1 m³ |
| 131495 | FEE SERVICE WAITING TIME PER MINUTE | 95 /M |
| 140066 | FEE SERVICE RETURN PER KM PER CU M | 2 km |
| 159028 | FEE SERVICE ENVIRONMENTAL CONC DISPOSAL | 4 m³ |
| 257145 | TRANSPORT CHARGE | 2 m³ |
| 231299 | WAITING TIME FEE WORKING HOURS | 75 /M |
| Product and additives |  |  |
| 256219 | ENVISIA HIGH SLUMP 40MPA 20MM 150MM | 2 m³ |
| Driver Instructions |  |  |
| DANIEL 0423291890 |  |  |
| Account charged |  |  |

Account Name	DEMARTIN & GASPARINI-L370

Customer Account Code	503098

Customer Name	DEMARTIN & GASPARINI-L370

Customer Code	29948

Project Name	ONE CIRCULAR QUAY - TOWER A

Project Code	Q-00141017

Purchase Order Number	J0805 - A

Abridged Conditions of Quotation and Supply - Concrete

'- r.

•

CLIENT	..,., . ""'-'"'•-·- ·· -	""''	.	·. •.	,.· . -- .	.	 -	""

_;_I I 2fj.

i	J\( ':" (H' r{	:SIA --d\F.'1 t V.	'	rsr r._i	(t'"'	:	a  "<;'. c::::,

JOB	I l LLL	L  --= -		.,l,'.6,V--J Li ....	r=i(=-.\	-

'2	DOCKIT NUMBER

CRUSHING

_.)I ...2, .L! L	l.o

-1C-{j;, C C: f\ C	\"\

mm	1 TIME BrTCil	s

DATE

J'; N I1tLICH;t;1· o,·li.----=",:=,;;. ,---

ALMS ·	C0N.C.

··R·J·>:;C,.7I ·1	TEMPS	I f[Ai

•

SPEC

DIAMETER	CRUSHING

STRENG;;   L2l8J	•

I

SAMPLING	•

COMPACT L	L  CURE

DATE

1115J-15Jibt, ,I .JS-I W2@l

lc)11I	,,·-,.-,'  I

INAf

J_

POUR LOCA

|  | • |  |  |  |
| --- | --- | --- | --- | --- |
| • |  |  |  |  |

I	I

I	I	I

MAX, AND MIN CURING TEMP. MUST BE RECORDED IF FIELD CURING IS GREATER THAN 36 HOURS	DATE	i IN!T-1

SAMPLING ANDTESTING IS PERFORMED TO A.S.1012 PARTS 1, 3.1, 8.1, 8.2, 9, 12.1, 12.2, 13

SAMPLING:	1 AGITATOR AT DISCHARGE, 2 SITE SAMPLED, 3 OTHER-SPECIFY

COMPACTION:  1 RODDING, 2 INTERNAL VIBRATION, 3 EXTERNAL VIBRATION, 4 RAMMING	'-....1.-.J..c..J.Jc...;..L.-'-'---'-   -.J	·	1

CAPPING:	1 SULPHUR, 2 RUBBER, 3 OTHER-SPECIFY	-

LABORATORY:  1 ARTARMON, 2WESTERN

SLUMP LIMITS: 1 NOMINAL, 2 MAXIMUM	SIGNATORY CHECK:	-	-	-	- 	,

- "-

=

FIELDTEST SHEET

I

□

,fREOUEST CONTROL

MAX LOAD

lit)

lL-

w

□YES	•

DIAMETER

=

SAMPLINGL I--FL COMPA,-,- -	CURE

CA-  S CONC.I f[A

REMARKS I

MAX. AND MIN CURING TEMP.MU.ST BE RECORDED IF FIELD CURING IS GREATERTRAN 36 HOORS SAMPLIN_G ANDTESTING IS PERFORMED TO A.S_1012 PARTS 1, 3.1, 8.1, 8.2, 9, 12.1, 12.2, 13

SAMPLING:	1 AGITATOR AT DISCHARGE, 2 SITE.SAMPLED, 3 OTHER-SPECIFY

COMPACTION:  1 RODDING, 2 INTERNAL VIBRATION, 3 EX'TEANAL VIBRATION, 4 RAMMING

CAPPING:	1 SIULPHUR, 2 A'UBBER, 3 OTHER-SPECIFY

LABORATORY:  1 ARTARMON, 2 WESTERN

SJUMP LIMIJS: ,UJOMINAL, 2 MAXIMUM	SIGNATORY CHECK:	I.L--.!

-•

-:Ml c:'srr/ _ .P--1 L	.1

.	==:I .'"-  ;==--+;-:  =:f..'if:',	...